Supporting Information Document

Protocol 7: Automated Synthesis of Chitosan-Based Bio-Hydrogels for Biomedical and Formulation Research

Deeper Context, Scientific Intent, Automation Logic, and Webinar Explanation

1. Support Status on Protoly / NSL

This protocol is partially NSL-supported.

The main preparation workflow can be supported on NSL using available physical modules such as:

| NSL Module | Role in This Protocol |
| --- | --- |
| Reservoir Dispense | Dispensing water, chitosan solution, humectant, secondary polymer, stabilizer, or conditioning solution |
| Stirrer | Controlled mixing during hydrogel preparation |
| Heater | Mild temperature support for polymer hydration |
| Wait | Hydration, maturation, and stabilization time |
| Sterilization UV | Pre-process chamber sterilization |
| LED Illumination | White light support for visual observation |
| Camera | Visual recording of gel appearance |
| Exhaust | Chamber airflow support where required |
| Environment Sensors | Ambient chamber condition record |
| Sonicator | Optional dispersion/de-bubbling support |
| Sonicator Bath Heater | Optional mild bath temperature support during sonication |

The following are not NSL modules in the current step builder and should be treated as offline/manual/supporting steps only:

| Offline / External Step | Purpose |
| --- | --- |
| pH measurement | Confirmation of final hydrogel pH |
| Rheology / viscosity testing | Mechanical and flow property analysis |
| Swelling study | Water uptake and hydrogel swelling behaviour |
| Sterility testing | Microbiological safety assessment |
| Cytotoxicity testing | Biological compatibility screening |
| Drug release study | Payload release profile |
| Microscopy / FTIR / SEM / DLS | External material characterization |
| Final sample labelling | Manual record and storage action |

2. Purpose of This Supporting Information

This supporting document explains the deeper scientific and project-level intent behind the chitosan-based bio-hydrogel protocol. The main Protoly protocol already contains the direct fillable sections such as title, description, abstract, introduction, materials, method steps, methodology, discussion, and conclusion.

Therefore, this document is not meant to repeat the complete protocol step-by-step. Its purpose is to explain:

| Area | Explanation |
| --- | --- |
| Scientific context | Why chitosan hydrogels are useful |
| Formulation logic | Why each formulation variable matters |
| Automation relevance | How NSL-supported steps improve repeatability |
| Webinar value | How this protocol can be explained to participants |
| Limitations | What the protocol does not prove |
| Future scope | How this protocol can be expanded later |

This document can be uploaded in the Supporting Information section of Protoly to help readers understand the background, intent, and proper interpretation of the protocol.

3. Overall Concept of the Protocol

The basic idea of this protocol is to demonstrate that a hydrogel formulation, which is usually prepared manually, can be converted into a structured digital workflow.

A hydrogel is not simply a liquid mixture. It is a hydrated polymer network. Its final behaviour depends on how the polymer is dissolved, hydrated, mixed, stabilized, and matured. In manual formulation work, these details may change from batch to batch. For example, one operator may stir faster, another may add the polymer slowly, another may heat for a longer time, and another may leave the gel to mature overnight. These small differences can affect the final appearance and consistency of the hydrogel.

Protoly helps convert this type of formulation work into a defined workflow. NSL can then support the practical execution of selected steps such as liquid dispensing, stirring, heating, waiting, illumination, and camera documentation.

The purpose is not to claim that the machine alone produces a final biomedical product. The purpose is to show how an early-stage hydrogel preparation method can become more structured, repeatable, and easier to compare across batches.

4. Why Chitosan-Based Hydrogels Were Selected

Chitosan is a suitable polymer for this protocol because it is scientifically relevant, relatively easy to explain, and useful in several formulation-development areas.

Chitosan-based hydrogels are commonly explored in:

| Application Area | Relevance |
| --- | --- |
| Wound dressing research | Soft hydrated matrix, possible bioadhesive behaviour |
| Drug delivery models | Can hold or release model compounds |
| Biomedical material studies | Biopolymer-based material platform |
| Cosmetic and topical formulations | Gel-like texture and spreadability |
| Antimicrobial material research | Can be combined with nanoparticles or active agents |
| Tissue engineering models | Hydrated polymer scaffold concept |
| Educational demonstrations | Clear example of polymer hydration and gel formation |

For the Protoly webinar, chitosan hydrogel is a good example because the process is understandable even to general participants. Viewers can visually observe the change from solution to gel-like material, and the concept can be connected to product development, biomedical research, and formulation science.

5. Scientific Background

Hydrogels are polymeric systems that can retain a large amount of water. Their structure may be formed through physical entanglement, ionic interactions, hydrogen bonding, pH-based transitions, polymer blending, or chemical crosslinking, depending on the selected formulation.

In chitosan-based hydrogels, several factors influence the final gel behaviour:

| Factor | Effect on Hydrogel |
| --- | --- |
| Chitosan concentration | Influences thickness, viscosity, and gel strength |
| Solvent condition | Affects chitosan solubility and uniformity |
| pH-conditioning | Influences polymer charge and stability |
| Temperature | Supports hydration but may also affect viscosity |
| Stirring speed | Controls mixing uniformity and bubble formation |
| Hydration time | Allows polymer chains to swell and organize |
| Secondary polymer | Modifies texture, strength, and water retention |
| Humectant | Improves softness and moisture retention |
| Maturation time | Allows the gel matrix to stabilize |

This makes hydrogel preparation a good candidate for automation-assisted formulation development. The process contains several measurable and controllable variables, and the results can be compared between batches.

6. Intent of the Hydrogel Protocol

The intent of this protocol is to create a research-scale chitosan-based hydrogel prototype, not a final medical or commercial product.

The prepared hydrogel may be used as a starting material or demonstration model for:

| Purpose | Explanation |
| --- | --- |
| Hydrogel formulation training | Shows how polymer gels are prepared |
| Biomedical material concept | Demonstrates hydrated biopolymer systems |
| Drug delivery model | Can later include model payloads |
| Nanoparticle-loaded gel development | Can incorporate AgNPs, ZnO, iron oxide, or other dispersions |
| Topical formulation prototype | Useful for gel consistency and spreadability trials |
| Comparative formulation study | Different batches can be prepared and compared |
| Webinar demonstration | Easy to explain visually and practically |

The protocol should be communicated as a platform method. That means the same base hydrogel workflow can be modified for different experimental directions.

7. Difference Between This Protocol and Chitosan Nanoparticle Protocol

It is important to keep this protocol clearly different from the previous chitosan nanoparticle protocol.

| Point | Chitosan Nanoparticle Protocol | Chitosan Bio-Hydrogel Protocol |
| --- | --- | --- |
| Main output | Nanoparticle dispersion | Gel or semi-solid hydrogel matrix |
| Main mechanism | Ionic gelation / particle formation | Polymer hydration and gel network development |
| Main focus | Nanocarrier model | Hydrogel material / formulation matrix |
| Critical variable | Chitosan:TPP ratio | Polymer concentration, hydration, viscosity, gel maturation |
| Visual output | Turbidity / dispersion | Gel consistency / uniformity |
| Possible use | Drug delivery nanoparticle model | Hydrogel, topical matrix, dressing model, formulation base |
| NSL value | Controlled crosslinker addition and mixing | Controlled dispensing, stirring, heating, and maturation |

This distinction will help avoid repetition between Protoly protocols and make each protocol look independent.

8. Automation Logic Behind the Workflow

The automation value in this protocol comes from controlling the early preparation conditions.

In manual hydrogel preparation, several steps are subjective:

| Manual Issue | Automation-Assisted Improvement |
| --- | --- |
| Approximate liquid addition | Reservoir dispensing defines volume |
| Variable stirring | Stirrer provides defined RPM |
| Uncontrolled heating | Heater gives set temperature and time |
| Irregular waiting time | Wait step standardizes hydration and maturation |
| Poor documentation | Protoly records the defined sequence |
| Visual comparison difficult | Camera and LED illumination support batch documentation |

For example, if one batch is heated for 20 minutes and another for 40 minutes, the difference is clearly recorded. If one batch uses 300 RPM and another uses 450 RPM, the comparison becomes easier. This structured approach helps convert formulation work from guess-based trials into planned experiments.

9. Role of Each NSL-Supported Step

9.1 Sterilization UV

This step prepares the working chamber before formulation handling. It is not a replacement for full sterility validation, but it is useful for reducing environmental contamination during demonstration or research-scale work.

9.2 Reservoir Dispense

This is one of the most important modules for the protocol. It allows controlled addition of formulation components such as:

| Component | Function |
| --- | --- |
| Deionized water | Dilution and hydration medium |
| Chitosan solution | Main hydrogel-forming polymer |
| Glycerol / humectant | Softness and moisture support |
| Secondary polymer | Texture and matrix modification |
| Conditioning solution | pH/stability adjustment support |
| Model payload | Optional drug delivery or visualization model |

9.3 Stirrer

Stirring helps distribute the polymer and other components uniformly. However, excessive stirring may introduce air bubbles or disturb gel formation. For hydrogel workflows, gentle-to-moderate stirring is usually more useful than very high-speed mixing.

9.4 Heater

Mild heating can support polymer hydration and reduce viscosity-related non-uniformity. Heating should remain controlled because excessive temperature may cause drying, concentration change, or polymer instability.

9.5 Wait

The Wait module is important because hydrogel formation is not always immediate. The polymer phase may need time to hydrate, swell, and stabilize. Waiting periods are useful after initial mixing, after conditioning solution addition, and during final gel maturation.

9.6 LED Illumination and Camera

These modules support visual documentation. They can help record:

| Observation | Importance |
| --- | --- |
| Gel clarity | Indicates uniformity or turbidity |
| Bubbles | Shows mixing-related air entrapment |
| Lumps | Indicates poor polymer hydration |
| Phase separation | Indicates formulation instability |
| Flow behaviour | Gives preliminary handling idea |
| Colour uniformity | Useful if a marker or payload is added |

This is visual observation only. It should not be described as UV-Vis or optical absorbance measurement.

9.7 Environment Sensors

Environment sensors can record ambient chamber conditions. This is useful for batch records, especially when hydrogel behaviour may be affected by room temperature or humidity.

9.8 Sonicator and Sonicator Bath Heater

Sonication can be used cautiously if the formulation contains particles, aggregates, or non-uniform dispersion. For a mature gel, sonication may disturb the structure, so it should be optional and mild. Sonicator bath heating may support controlled bath temperature if sonication is used.

10. Key Formulation Components and Their Purpose

10.1 Chitosan Solution

Chitosan is the main polymer phase. It provides the base structure of the hydrogel. The concentration and quality of chitosan solution strongly affect the final gel.

Important considerations:

| Parameter | Effect |
| --- | --- |
| Chitosan concentration | Controls thickness and viscosity |
| Molecular weight | Affects strength and flow |
| Degree of deacetylation | Affects charge and interaction |
| Solvent system | Affects solubility |
| Pre-filtration | May reduce lumps |

10.2 Deionized Water

Water adjusts the working volume and hydration environment. Too much water may reduce gel strength, while too little may produce a thick or non-uniform matrix.

10.3 Glycerol or Humectant

Glycerol may improve softness, spreadability, and moisture retention. It can make the hydrogel more flexible and less brittle.

10.4 Secondary Polymer

A secondary polymer can modify the hydrogel structure. For example:

| Secondary Polymer | Possible Purpose |
| --- | --- |
| Sodium alginate | Improves gel network and water retention |
| HPMC | Improves viscosity and smoothness |
| Gelatin | Adds biological material relevance |
| Carbopol base | Improves topical gel consistency |
| PVA | Supports film-forming and hydrogel strength |

The choice depends on the final research direction.

10.5 Conditioning or Stabilizing Solution

A mild conditioning solution may be used to adjust the hydrogel environment. However, accurate pH confirmation should be done manually/offline. This step should be described carefully, without implying that NSL automatically measures or adjusts pH unless such a feature is added later.

11. Critical Parameters to Record

For good documentation, the following information should be recorded for every hydrogel batch:

| Parameter | Why It Matters |
| --- | --- |
| Batch ID | Traceability |
| Chitosan concentration | Main formulation variable |
| Chitosan volume | Affects final polymer content |
| Water volume | Affects dilution |
| Humectant volume | Affects softness and spreadability |
| Secondary polymer type | Affects gel structure |
| Heating temperature | Affects hydration |
| Heating duration | Affects viscosity development |
| Stirring RPM | Affects uniformity and bubbles |
| Hydration time | Affects gel formation |
| Maturation time | Affects final consistency |
| Visual appearance | Preliminary quality observation |
| Offline pH | Important for suitability and stability |
| Offline viscosity/rheology | Important for formulation comparison |

This type of record makes the protocol useful for systematic formulation development.

12. Expected Observations

The hydrogel may show different visual and physical characteristics depending on the formulation.

| Observation | Possible Meaning |
| --- | --- |
| Smooth uniform gel | Good polymer hydration and mixing |
| Lumps or clumps | Incomplete hydration or poor mixing |
| Many air bubbles | Stirring speed too high or poor degassing |
| Thin watery gel | Low polymer concentration or excess water |
| Very thick gel | High polymer concentration or insufficient dilution |
| Phase separation | Poor compatibility between components |
| Cloudiness | Polymer interaction or dispersion issue |
| Sticky texture | High chitosan or humectant effect |
| Poor spreadability | Gel too concentrated or under-conditioned |
| Stable appearance after rest | Better preliminary formulation stability |

These observations are useful for early screening but are not a replacement for rheology or stability testing.

13. Suggested Batch Comparison Table

This type of table can be used in supporting data or future documentation.

| Batch ID | Chitosan Level | Secondary Polymer | Humectant | Heating | Stirring RPM | Maturation Time | Visual Result | Remarks |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHG-01 | Low | None | Low | 35 °C / 20 min | 300 | 60 min | Thin gel | Needs higher polymer |
| CHG-02 | Medium | HPMC | Medium | 40 °C / 30 min | 350 | 90 min | Smooth gel | Good prototype |
| CHG-03 | High | Alginate | Medium | 40 °C / 30 min | 450 | 120 min | Thick with bubbles | Reduce RPM |
| CHG-04 | Medium | Gelatin | Low | 35 °C / 20 min | 300 | 90 min | Soft gel | Suitable for further testing |

14. Possible Formulation Variants

The same base protocol can be adapted into several different hydrogel directions.

14.1 Simple Chitosan Hydrogel

A basic hydrogel made using chitosan, water, and conditioning solution. Useful for teaching and baseline comparison.

14.2 Chitosan-HPMC Hydrogel

Useful for smoother topical-type gel texture and improved viscosity.

14.3 Chitosan-Alginate Hydrogel

Useful for stronger network formation and biohydrogel demonstration.

14.4 Chitosan-Gelatin Hydrogel

Useful for biomedical material and scaffold-related demonstrations.

14.5 Nanoparticle-Loaded Chitosan Hydrogel

Can incorporate silver nanoparticles, zinc oxide nanoparticles, iron oxide nanoparticles, or chitosan nanoparticles for advanced material studies.

14.6 Model Drug-Loaded Hydrogel

Can include a safe model compound such as methylene blue, fluorescein, curcumin, caffeine, or vitamin C for drug loading and release concept demonstration.

15. Webinar Explanation Strategy

For a webinar, this protocol can be explained in a very simple way:

“We are taking a normal hydrogel preparation method and converting it into a structured digital protocol. The machine can dispense liquids, stir the mixture, heat it mildly, wait for hydration, and record the final appearance. Some tests like pH, viscosity, swelling, and biological testing are done separately. This shows how Protoly can connect protocol writing, robotic execution, and formulation development.”

This explanation is useful because it avoids overclaiming and clearly separates:

| NSL-Supported Part | Offline / External Part |
| --- | --- |
| Dispensing | pH measurement |
| Stirring | Rheology |
| Heating | Swelling study |
| Waiting | Cytotoxicity |
| Camera documentation | Stability validation |
| Environment logging | Biological performance testing |

This honest explanation will make the webinar more credible.

16. How This Protocol Fits the Larger Protoly Library

This protocol is part of a broader set of Protoly workflows related to:

| Protocol Type | Example |
| --- | --- |
| Nanoparticle synthesis | AgNP, ZnO, iron oxide |
| Biopolymer formulation | Chitosan nanoparticles, chitosan hydrogel |
| Drug delivery models | Liposomes, chitosan nanoparticles, hydrogels |
| Bio-interface studies | Protein corona, antibody conjugates |
| Biosensing | Electrochemical glucose detection |
| Enzyme assays | Catalase activity assay |

The chitosan hydrogel protocol is especially useful because it connects formulation science with biomedical material development. It can also act as a base matrix for other protocols. For example, silver nanoparticles prepared in another protocol may be incorporated into this hydrogel to develop an antimicrobial gel prototype.

17. Interpretation of Results

The result of this protocol should be interpreted mainly at the prototype level.

A good hydrogel prototype may show:

| Desired Feature | Interpretation |
| --- | --- |
| Uniform appearance | Good mixing and hydration |
| Minimal bubbles | Suitable stirring and maturation |
| No visible phase separation | Better formulation compatibility |
| Stable consistency after rest | Better preliminary gel stability |
| Smooth texture | Better formulation quality |
| Controlled flow | Useful for topical or biomedical model |

A poor hydrogel prototype may show:

| Problem | Possible Cause |
| --- | --- |
| Lumps | Incomplete polymer hydration |
| Excessive bubbles | High stirring speed |
| Watery texture | Low polymer concentration |
| Overly thick texture | High polymer level or low water |
| Phase separation | Incompatible components |
| Colour non-uniformity | Poor mixing of marker or payload |
| Rapid drying | Insufficient humectant or poor storage |

This interpretation can guide the next formulation round.

18. Suggested Offline Characterization Plan

After NSL-assisted preparation, the hydrogel can be studied using offline tests.

| Test | Purpose |
| --- | --- |
| pH measurement | Confirms suitability for intended application |
| Viscosity/rheology | Measures flow and mechanical behaviour |
| Swelling study | Evaluates water uptake |
| Spreadability test | Useful for topical formulation |
| Stability observation | Checks phase separation over time |
| Microscopy | Observes matrix uniformity or particles |
| FTIR | Studies chemical interactions |
| Drug loading study | Checks model payload incorporation |
| Release study | Evaluates release behaviour |
| Antimicrobial assay | If antimicrobial hydrogel is developed |
| Cytotoxicity assay | For biomedical compatibility screening |

These tests should be described as separate evaluations, not as NSL module functions.

19. Safety Considerations

Although this is a research-scale protocol, basic safety practices should be followed.

| Safety Point | Explanation |
| --- | --- |
| Use PPE | Gloves, lab coat, and eye protection |
| Handle acidic solutions carefully | Chitosan may be prepared in dilute acidic medium |
| Label all samples | Avoid confusion between batches |
| Avoid clinical claims | Prototype only, not medical product |
| Avoid unsafe payloads | Use safe model compounds for demonstration |
| Dispose waste properly | Especially if nanoparticles or biological materials are added |
| Use exhaust where required | Helpful during acidic or volatile handling |
| Do not use on skin or wounds | Unless validated under proper regulatory conditions |

For public webinar language, it is better to say:
“This is a research and educational hydrogel prototype, not a validated biomedical product.”

20. Limitations

This protocol has a defined scope. It prepares a hydrogel prototype but does not prove final performance.

Important limitations:

- The protocol does not confirm final hydrogel pH automatically.

- The protocol does not measure viscosity or rheology.

- It does not confirm sterility.

- It does not prove wound-healing performance.

- It does not prove cytocompatibility.

- It does not confirm long-term storage stability.

- It does not establish drug release profile.

- It does not validate antimicrobial activity.

- It does not replace regulatory testing.

- It does not create a ready-to-use clinical or commercial product.

These limitations should be clearly mentioned because Protoly may include NSL-supported, partially supported, and non-supported protocols. In this case, the preparation is partially supported, while deeper testing remains offline.

21. Future Development Possibilities

This protocol can be expanded later into more advanced Protoly workflows.

| Future Protocol | Purpose |
| --- | --- |
| Chitosan hydrogel swelling study | To study water absorption |
| Drug-loaded chitosan hydrogel model | To study payload incorporation |
| Chitosan hydrogel release study | To study release behaviour |
| AgNP-loaded chitosan hydrogel | To develop antimicrobial gel prototype |
| ZnO-loaded chitosan hydrogel | To study UV-responsive or protective formulation |
| Chitosan-alginate hydrogel optimization | To improve matrix strength |
| Chitosan-HPMC topical gel prototype | To improve spreadability |
| Comparative manual vs NSL-assisted hydrogel preparation | To demonstrate automation advantage |
| Hydrogel stability screening | To evaluate storage behaviour |
| Hydrogel biocompatibility model | To connect with cell-based studies |

22. Suggested Supporting Information Summary

The chitosan-based bio-hydrogel protocol demonstrates how a polymer formulation method can be organized into a structured Protoly workflow and partially executed using the NSL platform. The automated portion mainly supports liquid dispensing, stirring, mild heating, waiting, illumination, camera documentation, and environmental logging. These steps help reduce manual variation and improve batch documentation during early formulation development.

The protocol is intended for research, education, and prototype development. It provides a base hydrogel matrix that can be modified with secondary polymers, humectants, stabilizers, nanoparticles, or model payloads. Important performance evaluations such as pH testing, rheology, swelling, sterility, cytotoxicity, antimicrobial activity, and release studies should be performed separately using offline methods.

Overall, this protocol is valuable because it connects hydrogel science, formulation development, protocol management, and automation-assisted preparation in a practical and visually demonstrable workflow.